IP Addresses

- Public IP addresses are provided by Internet Service Providers (ISPs) for a monthly fee

- IPv4 uses a 32-bit numbering system, allowing for 2^32 (approx. 4.29 billion) IP addresses

- IPv6 uses a 128-bit numbering system, supporting up to 2^128 (approx. 340 trillion) IP addresses, resolving the limitations of IPv4

Windows Firewall Defender

- This is a Host firewall (a host firewall is a program that ships as part of your system or is installed)

MAC Address

- MAC stands for Media Access Control

- A MAC address is a 12-character hexadecimal number split into two's and separated by colons (e.g., a4:c3:f0:85:ac:2d)

- The first 6 characters represent the manufacturer, and the last 6 are a unique identifier

Ping and ICMP

- Ping uses ICMP (Internet Control Message Protocol) packets to test the performance and reliability of a connection between devices

- Ping measures the time taken for ICMP echo packets to travel between devices and receive a reply

- Ping can be performed against devices on a network or websites using the command ping <IP address or URL>

LAN Topologies

- Star Topology: Devices are individually connected to a central networking device (switch or hub)

- Bus Topology: Devices are connected to a single backbone cable, which can cause bottlenecks and difficulty in troubleshooting

- Ring Topology: Devices are connected directly to each other in a loop, requiring less cabling and dedicated hardware

Network Devices

- Switch: Used to centrally connect multiple devices on a local network and transmit data to the correct location

- Router: Connects networks and passes data between them using routing

Subnetting

- Subnetting divides a network into smaller pieces

- There are 32 bits in a subnet mask, and each octet can range from 0-255

Network Addressing

- Network Address: Identifies the start of a network

- Host Address: Identifies devices within a network

- Default Gateway: The device responsible for sending data to another network

Address Resolution Protocol (ARP)

- ARP allows devices to identify themselves on a network by associating their MAC address with an IP address

- Devices keep a log of MAC addresses associated with other devices

- When a device wants to communicate with another, it sends a broadcast to the entire network searching for the specific device's MAC address

- If a device has the requested IP address, it sends an ARP reply with its MAC address

- The initial device stores this MAC address and IP address mapping in its ARP cache

IP addresses can be assigned in two ways:

- Manual Assignment

  - IP addresses are physically entered into a device.

- Automatic Assignment (Most Common)

  - A DHCP (Dynamic Host Configuration Protocol) server is used.

  - When a device connects to a network and does not have a manually assigned IP address, it follows these steps:

    - The device sends a DHCP Discover request to check for DHCP servers on the network.

    - The DHCP server responds with a DHCP Offer, suggesting an IP address for the device to use.

    - The device sends a DHCP Request, confirming its acceptance of the offered IP address.

    - The DHCP server sends a DHCP ACK (Acknowledgment), allowing the device to start using the assigned IP address.

OSI (Open Systems Interconnection) Model

- Fundamental, critical model in networking

- Provides framework for how networked devices send, receive, interpret data

- Benefit: Allows devices with different functions/designs to communicate on a network

- Data sent follows OSI model uniformity, understandable by other devices

7 Layers of OSI Model (Layer 7 to Layer 1):

Key Points:

- Each layer has different responsibilities

- As data travels through layers, specific processes occur

- Information is added to data (encapsulation process)

Layer 7: Application Layer

- Most familiar layer to users

- Provides protocols and rules for user interaction with data (send/receive)

- Examples:

  - Email clients

  - Web browsers

  - File transfer software (e.g., FileZilla)

  - Provide Graphical User Interface (GUI) for user interaction

- Other protocols:

  - DNS (Domain Name System)

    - Translates website addresses to IP addresses

Layer 6: Presentation Layer

- Standardization of data begins

- Acts as a translator for data to/from Application Layer (Layer 7)

- Allows different software/formats to communicate

  - Example: Different email clients can display email content consistently

- Handles data encryption and security features

  - HTTPS (secure website) encryption occurs at this layer

Layer 5: Session Layer

- Establishes connection (session) between computers for data transfer

- Session remains active while connection is live

Responsibilities:

- Synchronizes computers to ensure readiness before data exchange

- Divides data into smaller chunks (packets) for transfer

  - Beneficial for recovering from connection loss (only resend remaining chunks)

- Sessions are unique - data cannot travel across different sessions

Key Points:

- Manages sessions between computers

- Prepares data for transfer by dividing into packets

- Improves reliability by allowing partial retransmission if needed

Layer 4: Transport Layer

- Vital for transmitting data across a network

- Uses one of two protocols based on specific factors:

  - TCP (Transmission Control Protocol)

    - Designed for reliability and guarantee

    - Establishes constant connection between devices for data transfer

    - Incorporates error checking

      - Ensures data chunks from Session Layer are received and reassembled in order

    - Used for services that require accuracy and completeness

      - File sharing, web browsing, email, etc.

In the diagram below, we can see how a picture of a cat is broken down into small pieces of data (known as packets) from the "webserver", where the "computer" re-constructs the picture of the cat into the correct order.

  - UDP (User Datagram Protocol)

    - Less advanced than TCP

    - No error checking or reliability features

    - Data is sent without synchronization or guarantee of delivery

    - Useful for small data transfers and situations where some data loss is acceptable

- Device discovery protocols like ARP, DHCP

- Video streaming (some pixelation is okay)

Layer 3: Network Layer

- Where routing and reassembly of data chunks (packets) occur

- Routing determines optimal path for packet delivery

Routing Factors:

- Shortest path (fewest devices to cross)

- Most reliable path (least packet loss)

- Fastest physical connection (fiber > copper)

Routing Protocols:

- OSPF (Open Shortest Path First)

- RIP (Routing Information Protocol)

- Determine optimal routes based on factors above

IP Addressing:

- Operates using IP addresses (e.g. 192.168.1.100)

- Devices like routers capable of IP routing = Layer 3 devices

Key Points:

- Handles routing and reassembly of packets

- Routing protocols find optimal delivery paths

- Uses IP addressing for packet delivery

- Layer 3 devices (routers) operate at this layer

Layer 2: Data Link Layer

- Focuses on physical addressing for data transmission

- Receives packets from Network Layer (with IP address)

- Adds physical MAC (Media Access Control) address of receiving endpoint

MAC Addressing:

- Network Interface Card (NIC) has unique MAC address

- MAC addresses set by manufacturer and burned into NIC

- Cannot be changed (but can be spoofed)

- Physical MAC address used to identify where to send data

Responsibilities:

- Present data in suitable format for transmission

- Package data into frames for sending over physical layer

Key Points:

- Handles physical addressing using MAC addresses

- Obtains IP addresses from Network Layer and adds MAC addresses

- Prepares data for transmission over the Physical Layer

Layer 1 Physical- For example ethernet cables

Packets and Frames

- Small pieces of data that form a larger message/information

Encapsulation

- Process discussed in OSI model

- Packet: Contains IP addresses

- Frame: Data inside packet after encapsulating info stripped away

Advantages of Packets

- Efficient communication across networked devices

- Less chance of bottlenecking than sending large messages at once

- Example: Loading website image - sent in small pieces, reconstructed on computer

Ports

- Essential for data exchange

- Analogy: Ships docking at compatible ports in a harbor

- Ports enforce compatibility rules for devices communicating

Port Numbers

- Numerical values between 0 and 65535

- Risk of losing track of applications using each port

Standard Port Assignment

- Associate applications/software with standard port numbers

- Example: Web browsers use port 80

- Allows different software to interpret data consistently

Common Ports

- Ports between 0 and 1024 are common ports

- Standard rules assigned for common protocols/applications

To connect to a port such as 1234 using the IP address 8.8.8.8 you use this command:

nc 8.8.8.8 1234

Firewalls:

Operate at layers 3 and 4. A stateful firewall inspects the entire connection while the stateless firewall inspects the individual packets.

# VPN Basics

## What is a VPN?

- A Virtual Private Network (VPN) is a technology that allows devices on separate networks to communicate securely by creating a dedicated path (tunnel) between them over the internet.

- Devices connected within this tunnel form their own private network.

## Benefits of a VPN

- Allows connecting networks in different locations

  - Useful for businesses with multiple offices to access resources like servers/infrastructure remotely.

- Offers privacy

  - Uses encryption to protect data, making it only understandable between the source and destination devices.

  - Protects traffic on public WiFi networks where no encryption is provided.

- Offers anonymity

  - Used by journalists and activists to safely report in countries with controlled freedom of speech.

  - Hides traffic from ISPs and intermediaries, but anonymity level depends on VPN logging practices.

- Secure access to TryHackMe vulnerable machines

  - Allows secure interaction with machines without making them directly accessible on the internet.

  - Prevents ISPs from thinking you're attacking machines on the internet.

  - Provides security to TryHackMe by not exposing vulnerable machines directly.

## VPN Technologies

- PPP (Point-to-Point Protocol)

  - Used by PPTP for authentication and data encryption.

  - Uses private keys and public certificates for connection.

  - Non-routable (can't leave a network by itself).

- PPTP (Point-to-Point Tunneling Protocol)

  - Allows PPP data to travel and leave a network.

  - Easy to set up and supported by most devices.

  - Weakly encrypted compared to alternatives.

- IPSec (Internet Protocol Security)

  - Encrypts data using the existing Internet Protocol (IP) framework.

  - Difficult to set up but offers strong encryption if successful.

  - Supported on many devices.

# What is a Router?

- Job: Connect networks and pass data between them

- Uses routing process to create paths for data to travel across networks

- Operates at Layer 3 (Network Layer) of OSI model

- Interactive interface for configuration (port forwarding, firewalling, etc.)

- Dedicated devices, different from switches

- Routing protocols decide optimal path based on factors like shortest path, reliability, medium speed

# What is a Switch?

- Dedicated networking device for connecting multiple devices

- Operates at Layer 2 (Data Link Layer) or Layer 3 (Network Layer) of OSI model

## Layer 2 Switches

- Forward frames using MAC addresses

- Solely responsible for sending frames to correct devices

## Layer 3 Switches

- More sophisticated than Layer 2

- Can perform some router responsibilities

- Forward frames to devices (like Layer 2)

- Route packets to other devices using IP protocol

# VLANs (Virtual Local Area Networks)

- Technology to virtually split devices within a network

- Provides network separation and security

- Devices can share Internet connection but are treated separately

- Rules determine how specific devices communicate with each other

- Example: Sales and Accounting departments can access Internet but not communicate with each other